The electron temperature enhancement produced by the HF heating in the F2 region

Getmanov I.K.1, Li J.2

Master student, 2nd year

1Saint Petersburg State University,

Faculty of Physics, Saint Petersburg, Russia

2Harbin Institute of Technology,

Harbin, China

E-mail: igor.getmanov1@gmail.com

The new standards of radio communication demand the improvement of the quality of signal transmission through such a complex medium as the ionosphere. The high-power high frequency (HF) electromagnetic wave propagation in ionospheric plasma is associated with a variety of induced instabilities [1]. It is assumed that Langmuir turbulence generated by HF radio waves in the plasma resonance area is responsible for the efficient electrons acceleration, thus, leading electron energy distribution function to be non-equilibrium [2]. This increase in the suprathermal population can substantially change the macroscopic parameters of ionospheric plasma (electron temperature, density, etc.) [2,3], which is of great interest for both fundamental study and HF communication applications.

This work is devoted to the study of the electron temperature dynamics of F-region ionospheric plasma in the high-power alternating electric field. In the following, the electron energy balance equation has been formulated. The impact of the vibrational excitation of N2 and O(1S), O(1D) excitations to the electron cooling as sources of inelastic collisions have been taken into account. Thermal electron heating has been introduced in the Landau collision integral form[4] with a stationary flow of the suprathermal electrons of various EEDFs, namely, bimaxwellian, power law, complex distribution with electron depleted areas [5]. The fast electron’s effective temperature is assumed to be within 5 – 10 eV energy range.

The study showed that the temperature of thermal (Maxwellian) electrons could be increased by 0.5 to 1.0 eV depending on the suprathermal electron density. This effect caused by the high-power HF radio wave heating demonstrates the nonlinearity of the propagation problem of the alternating field of that kind that should be taken into account.

Асknowledgements

Authors are grateful to our supervisors Anatoly Kudryavtsev and Chengxun Yuan, for their assistance, remarks, and fruitful discussions of the topic.

References

1. Гуревич А.В. Нелинейные явления в ионосфере // Успехи Физических Наук, 2007, No 177, С. 1145–1177, doi: 10.3367/UFNr.0177.200711a.1145

2. Gurevich A.V. Langmuir Turbulence in Ionospheric Plasma // Plasma Physics Reports, Vol. 30, No. 12, 2004, pp. 995–1005.

3. Vlasov M.N., Kelley M.C., Gerken E.A. On the energy distribution of suprathermal electrons produced by HF heating in the F2 region // Journal of Atmospheric and Solar-Terrestrial Physics, Vol. 67, 2005, pp. 405-412, doi: 10.1016/j.jastp.2004.08.006

4. Yuan. Ch., Bogdanov E.A., Eliseev S.I., Kudryavtsev A.A. 1D kinetic simulations of a glow discharge in helium // Physics of Plasmas, Vol. 24, 073507 (2017), doi: 10.1063/1.4990030

5. Vlasov M.N., Kelley M.C., Hysell D.L. The main types of electron distribution determined by model fitting to optical emissions during HF wave ionospheric modification experiments // Journal of Geophysical Research: Space Physics, Vol. 118, 2013, pp. 3877-3890. doi: 10.1002/jgra.50364